Khoi Duong

Prof. Yang

CE450L

10/26/2022

LAB#5

GitHub link: https://github.com/MynameisKoi/CE450L/tree/main/Lab%235

7_segment.py ⇔ Display on the 7-segment display

Breadboard setup:

Source code: https://github.com/MynameisKoi/CE450L/blob/main/Lab%235/7_segment.py

#!/usr/bin/env python3

import RPi.GPIO as GPIO

import time

from sys import version_info

if version_info.major == 3:

raw_input = input

# Set up pins

SDI   = 17

RCLK  = 18

SRCLK = 27

# Define a segment code from 0 to F in Hexadecimal

# Common cathode

segCode = [0x3f,0x06,0x5b,0x4f,0x66,0x6d,0x7d,0x07,0x7f,0x6f,0x77,0x7c,0x39,0x5e,0x79,0x71]

# Common anode

# segCode = [0xc0,0xf9,0xa4,0xb0,0x99,0x92,0x82,0xf8,0x80,0x90,0x88,0x83,0xc6,0xa1,0x86,0x8e]

def print_msg():

print ("========================================")

print ("|         Segment with 74HC595         |")

print ("|    ------------------------------    |")

print ("|         SDI connect to GPIO17        |")

print ("|         RCLK connect to GPIO18       |")

print ("|        SRCLK connect to GPIO27       |")

print ("|                                      |")

print ("|     Control segment with 74HC595     |")

print ("|                                      |")

print ("|                            SunFounder|")

print ("========================================")

print ("Program is running...")

print ("Please press Ctrl+C to end the program..")

raw_input ("Press Enter to begin\n")

def setup():

GPIO.setmode(GPIO.BCM)

GPIO.setup(SDI, GPIO.OUT, initial=GPIO.LOW)

GPIO.setup(RCLK, GPIO.OUT, initial=GPIO.LOW)

GPIO.setup(SRCLK, GPIO.OUT, initial=GPIO.LOW)

# Shift the data to 74HC595

def hc595_shift(dat):

for bit in range(0, 8):

GPIO.output(SDI, 0x80 & (dat << bit))

GPIO.output(SRCLK, GPIO.HIGH)

time.sleep(0.001)

GPIO.output(SRCLK, GPIO.LOW)

GPIO.output(RCLK, GPIO.HIGH)

time.sleep(0.001)

GPIO.output(RCLK, GPIO.LOW)

def main():

print_msg()

while True:

# Shift the code one by one from segCode list

for code in segCode:

hc595_shift(code)

print ("segCode[%s]: 0x%02X"%(segCode.index(code), code)) # double digit to print

time.sleep(0.5)

def destroy():

GPIO.cleanup()

if __name__ == '__main__':

setup()

try:

main()

except KeyboardInterrupt:

destroy()

Run code & demonstration:

Video link: https://youtu.be/nybU3K42A50

dice.py ⇔ Rolling dice with 7-segment display

Breadboard setup:

Source code: https://github.com/MynameisKoi/CE450L/blob/main/Lab%235/dice.py

#!/usr/bin/env python3

import RPi.GPIO as GPIO

import time

import random

from sys import version_info

if version_info.major == 3:

raw_input = input

# Set up pins

SDI   = 17

RCLK  = 18

SRCLK = 27

TouchPin = 22

# Define a segment code from 1 to 6 in Hexadecimal

SegCode = [0x06, 0x5b, 0x4f, 0x66, 0x6d, 0x7d]

# Used to record button press

flag = 0

def print_msg():

print ("========================================")

print ("|               Dice                   |")

print ("|    ------------------------------    |")

print ("|        SDI connect to GPIO17         |")

print ("|        RCLK connect to GPIO18        |")

print ("|       SRCLK connect to GPIO27        |")

print ("|     Button Pin connect to GPIO22     |")

print ("|                                      |")

print ("|     Control segment with 74HC595     |")

print ("|           random number 1~6          |")

print ("|  Press to suspend segment 2 seconds  |")

print ("|                                      |")

print ("|                            SunFounder|")

print ("========================================")

print ("Program is running...")

print ("Please press Ctrl+C to end the program...")

raw_input ("Press Enter to begin\n")

def setup():

GPIO.setmode(GPIO.BCM)

GPIO.setwarnings(False)

GPIO.setup(SDI, GPIO.OUT, initial=GPIO.LOW)

GPIO.setup(RCLK, GPIO.OUT, initial=GPIO.LOW)

GPIO.setup(SRCLK, GPIO.OUT, initial=GPIO.LOW)

GPIO.setup(TouchPin, GPIO.IN, pull_up_down = GPIO.PUD_UP)

GPIO.add_event_detect(TouchPin, GPIO.RISING, callback = randomISR, bouncetime = 20)

# Shift the data to 74HC595

def hc595_shift(dat):

for bit in range(0, 8):

GPIO.output(SDI, 0x80 & (dat << bit))

GPIO.output(SRCLK, GPIO.HIGH)

time.sleep(0.001)

GPIO.output(SRCLK, GPIO.LOW)

GPIO.output(RCLK, GPIO.HIGH)

time.sleep(0.001)

GPIO.output(RCLK, GPIO.LOW)

def randomISR(channel):

global flag

flag = 1

def destroy():

GPIO.cleanup()

def main():

global flag

print_msg()

while True:

num = random.randint(1,6)

hc595_shift(SegCode[num-1])

print (num, hex(SegCode[num-1]))

if flag == 1:

print ("Num: ", num)

time.sleep(2)

flag = 0

else:

time.sleep(0.01)

if __name__ == '__main__':

setup()

try:

main()

except KeyboardInterrupt:

destroy()

Run program & demonstration:

Video link: https://youtu.be/urOZOrzTO7w

For this question, we have to use the 4-digit 7-segment display.

We have the schema for the 4-digit 7-segment display as below:

Breadboard setup:

We have the code 1 as OFF and 0 as ON for the 7-segment display

The display follows the following format: (DP)GFEDCBA

We have the following display:

Thus, the hex code for numbers from 0 - 25 and a - z will be as follow:

Source code: https://github.com/MynameisKoi/CE450L/blob/main/Lab%235/4_7_segments.py

import RPi.GPIO as GPIO

import time

import threading

from sys import version_info

if version_info.major == 3:

raw_input = input

SDI = 24

RCLK = 23

SRCLK = 18

placePin = (10, 22, 27, 17)

number = (0xc0, 0xf9, 0xa4, 0xb0, 0x99, 0x92, 0x82, 0xf8, 0x80, 0x90)

alphabet = (0x88, 0x83, 0xa7, 0xa1, 0x86, 0x8e, 0xc2, 0x8b,

0xcf, 0xe1, 0x8a, 0xc7, 0xd4, 0xab, 0xa3, 0x8c, 0x98, 0xaf,

0x92, 0x87, 0xe3, 0xd5, 0x81, 0xb6, 0x91, 0x92, 0xff)

counter = 0

timer1 = 0

def clearDisplay():

for i in range(8):

GPIO.output(SDI, 1)

GPIO.output(SRCLK, GPIO.HIGH)

GPIO.output(SRCLK, GPIO.LOW)

GPIO.output(RCLK, GPIO.HIGH)

GPIO.output(RCLK, GPIO.LOW)

def hc595_shift(data):

for i in range(8):

GPIO.output(SDI, 0x80 & (data << i))

GPIO.output(SRCLK, GPIO.HIGH)

GPIO.output(SRCLK, GPIO.LOW)

GPIO.output(RCLK, GPIO.HIGH)

GPIO.output(RCLK, GPIO.LOW)

def print_msg():

print ("========================================")

print ("|          Display 1-25 and a-z        |")

print ("|    ------------------------------    |")

print ("|        SDI connect to GPIO24         |")

print ("|        RCLK connect to GPIO23        |")

print ("|        SRCLK connect to GPIO18       |")

print ("|                                      |")

print ("|  Control 7-seg display with 74HC595  |")

print ("|                                      |")

print ("|                            SunFounder|")

print ("========================================")

print ("Program is running...")

print ("Please press Ctrl+C to end the program...")

raw_input ("Press Enter to begin\n")

def pickDigit(digit):

for i in placePin:

GPIO.output(i,GPIO.LOW)

GPIO.output(placePin[digit], GPIO.HIGH)

def timer():

global counter

global timer1

timer1 = threading.Timer(0.5, timer)

timer1.start()

print("%d" % counter)

if counter > 51:

counter = 1

else:

counter += 1

def main():

global counter

print_msg()

global timer1

timer1 = threading.Timer(0.5, timer)

timer1.start()

while True:

if counter < 26:

clearDisplay()

pickDigit(0)

hc595_shift(number[counter % 10])

clearDisplay()

pickDigit(1)

hc595_shift(number[counter % 100//10])

else:

clearDisplay()

pickDigit(0)

hc595_shift(alphabet[counter - 26])

def setup():

GPIO.setmode(GPIO.BCM)

GPIO.setup(SDI, GPIO.OUT)

GPIO.setup(RCLK, GPIO.OUT)

GPIO.setup(SRCLK, GPIO.OUT)

for i in placePin:

GPIO.setup(i, GPIO.OUT)

def destroy():   # When "Ctrl+C" is pressed, the function is executed.

global timer1

GPIO.cleanup()

timer1.cancel()  # cancel the timer

if __name__ == '__main__':

setup()

try:

main()

except IndexError:

destroy()

except KeyboardInterrupt:

destroy()

Run code & stimulation:

Video link: https://youtu.be/sI9M2D-p-i4