Khoi Duong

Prof. Yang

CE450L

10/12/2022

QUIZ#1

GitHub link: https://github.com/MynameisKoi/CE450L/tree/main/Quiz%231

Breadboard setup:

Source code: https://github.com/MynameisKoi/CE450L/blob/main/Quiz%231/1.py

#!/usr/bin/env python3

import RPi.GPIO as GPIO

import time

from sys import version_info

if version_info.major == 3:

raw_input = input

# Set #17 as buzzer pin

BeepPin = 17

def print_message():

print ("========================================")

print ("|                 Beep                 |")

print ("|    ------------------------------    |")

print ("|        Buzzer connect to GPIO17      |")

print ("|                                      |")

print ("|            Make Buzzer beep          |")

print ("|                                      |")

print ("|                            SunFounder|")

print ("======================================\n")

print ("Program is running...")

print ("Please press Ctrl+C to end the program...")

raw_input ("Press Enter to begin\n")

def setup():

# Set the GPIO modes to BCM Numbering

GPIO.setmode(GPIO.BCM)

# Set LedPin's mode to output,

# and initial level to High(3.3v)

GPIO.setup(BeepPin, GPIO.OUT, initial=GPIO.HIGH)

def main():

print_message()

while True:

# Buzzer on (Beep)

print ("Buzzer On")

GPIO.output(BeepPin, GPIO.LOW)

time.sleep(0.1)

# Buzzer off

print ("Buzzer Off")

GPIO.output(BeepPin, GPIO.HIGH)

time.sleep(0.2)

def destroy():

# Turn off buzzer

GPIO.output(BeepPin, GPIO.HIGH)

# Release resource

GPIO.cleanup()

# If run this script directly, do:

if __name__ == '__main__':

setup()

try:

main()

# When 'Ctrl+C' is pressed, the child program

# destroy() will be  executed.

except KeyboardInterrupt:

destroy()

Run code and demonstration:

Video link:

https://github.com/MynameisKoi/CE450L/blob/main/Quiz%231/Demonstrations/1.mp4

We know that the LED is usually set up with a 220Ω resistor and a power supply of 3.3V

Therefore, for the 5V power supply, the resistor should have: 220*(5/3.3)≈330Ω

Breadboard setup:

Source code: https://github.com/MynameisKoi/CE450L/blob/main/Quiz%231/2.py

#!/usr/bin/env python3

import RPi.GPIO as GPIO

import time

from sys import version_info

if version_info.major == 3:

raw_input = input

# Set #17 as buzzer pin and #18 as LED pin

BeepPin = 17

LedPin = 18

def print_message():

print ("========================================")

print ("|                 Beep                 |")

print ("|    ------------------------------    |")

print ("|        Buzzer connect to GPIO17      |")

print ("|         LED connect to GPIO18        |")

print ("|                                      |")

print ("|            Make Buzzer beep          |")

print ("|             and LED blink            |")

print ("|                            SunFounder|")

print ("======================================\n")

print ("Program is running...")

print ("Please press Ctrl+C to end the program...")

raw_input ("Press Enter to begin\n")

def setup():

# Set the GPIO modes to BCM Numbering

GPIO.setmode(GPIO.BCM)

# Set LedPin and BeepPin's mode to output,

# and initial level to High(5v)

GPIO.setup(BeepPin, GPIO.OUT, initial=GPIO.HIGH)

GPIO.setup(LedPin, GPIO.OUT, initial=GPIO.HIGH)

def main():

print_message()

while True:

# Buzzer on (Beep)

print ("Buzzer & LED On")

GPIO.output(BeepPin, GPIO.LOW)

GPIO.output(LedPin, GPIO.LOW)

time.sleep(0.2)

# Buzzer off

print ("Buzzer & LED Off")

GPIO.output(BeepPin, GPIO.HIGH)

GPIO.output(LedPin, GPIO.HIGH)

time.sleep(0.2)

def destroy():

# Turn off buzzer and LED

GPIO.output(BeepPin, GPIO.HIGH)

GPIO.output(LedPin, GPIO.HIGH)

# Release resource

GPIO.cleanup()

# If run this script directly, do:

if __name__ == '__main__':

setup()

try:

main()

# When 'Ctrl+C' is pressed, the child program

# destroy() will be  executed.

except KeyboardInterrupt:

destroy()

Run code & demonstration:

Video link:

https://github.com/MynameisKoi/CE450L/blob/main/Quiz%231/Demonstrations/2.mp4